Instruction:

1st Floor top & bottom plates & 2nd Floor top & bottom plates:

Grab 1st-floor wall studs with the layout images from the pile, space them with enough room to fit studs between, and try to align the marks on the plates.

If the wall includes a window or door opening, those components should be installed first.

Jack Studs:

Grab the jack studs from the pile and nail them in place by aligning each one with the jack-stud layout marks on the 1st-floor walls.

Headers, Sills:

Grab the correct header or sill piece and install it by fitting it between the king studs and nail them at the opening shown in the layout. (King studs are the same size as standard studs.)

Special components:

Set solid header firmly in place, ensuring it has full bearing and is level. Secure the header with proper nailing. (The solid header has a 1/2-inch insulation board sandwiched inside.)

Install partition studs at locations where interior walls will connect to exterior walls, ensuring they are placed at the correct layout point. (The blocks must be labeled down or up) (Partition studs’ middle part may be built using scrap material, as long as all three pieces are at least 12 inches in length.)

Install full studs at required structural locations, ensuring all three studs are tightly aligned, flush with the plates, and nailed together securely. Verify the assembly is plumb and positioned exactly on the correct layout mark.

Door Headers:

Place door headers at the exact door opening height. Confirm level and proper length so they align with king and jack studs on each side.

Cripples:

Grab the cripples and nail them above the header or below the sill by aligning each piece with the spacing shown on the layout image.

California corners and Block corners:

Grab the labeled assemblies and attach them to the wall by setting each piece at its marked position on the layout.

Standard Studs:

Grab standard studs and install them by standing each one vertically on the plate and nailing them at the layout marks at the correct spacing.

Top Plate:

Grab the top plates and assemble them by placing each plate over the stud tops nailing through to tie the wall together.

At no time should you allow the top plate you are installing to extend past that mark. If it is short by an eighth of an inch, that is acceptable but never allows it to extend beyond the mark.

Top Plate Nailing Guide:

Stagger nails along the top plates—alternating up and down—at roughly every 16 inches instead of placing them in a straight line.

Labels (Pair with CAD drawing):

- 1st floor plates

- 2nd floor plates

- Jack studs

- Headers

- Sills

- Special components: Solid headers, Partition Studs, Full Studs etc.

- Door headers

- Cripples

- California corners

- Block corners

- Standard studs

- Top plate